- Purpose: This checklist evaluates attainment of vehicle radio equipment requirements for RACES operators to be deemed QUALIFIED FOR MUTUAL AID DEPLOYMENT anywhere within the National Capitol Region. Your personal “Go Kit” will be inspected separately using the Personal Equipment Checklist for 12-Hour Operational Period – Urban Environment, for each incident or exercise.

- A passing score is 20 points. Requirements listed in bold type may not be waived.

- Operator Name _____________________Call_________ Date___________

- Evaluator Name_____________________Call_________ Title___________

- Vehicle Make________________ Model _________ TAG_______________

- A separate inspection is required to be filed for each vehicle to be issued a RACES decal.

- □ 2m FM mobile installed with low power  < 5 w, high power ≥ 25 w.		+ 5 pts.	____

- Use of a brick amp or docking booster with an HT does not meet the requirement for mutual aid deployments.

- □ Ten memory channels are required as minimum.				+ 5 pts.	____

- Operator shall demonstrate that the radio is programmed to the RACES Communication Plan as a minimum.

- □ Continuous Tone Coded Squelch Encode is required.			+ 5 pts.	____

- Operator shall demonstrate the ability to program a new CTCSS tone and frequency.

- □ 2m radio, if frequency agile, field programmable from control pane		+ 3 pts.____

- Operator shall demonstrate the ability to program a new CTCSS tone and frequency.

- □ Continuous Tone Coded Squelch Decode, if capable				+ 1 pt.____

- □ Hard-wired, fused power connections to vehicle battery, each radio 		+ 1 pt.____

- □ Permanent antenna mounts on vehicle, 1 pt. each, maximum  4 points		+        ____

- □ Mobile 220 or 440 FM @ 10w minimum, if capable, per band			+ 1 pt.____

- □ Mobile SSB  @25w if assigned as a net liaison, Tech license either 2m, or 10m,  Gen. Lic. minimum 40m+75m, with 2m, 10m and 60m desired , 1 pt. per band, max. 5 pts.	+	____

- □ Portable(s) NOT used as primary radio in vehicle, IF  ≥ 2 watts, ten memories per band with CTCSS,  capable of being powered from three sources, the OEM battery pack, AA batteries and external 12V DC, on 2m, 220 or 440 FM , per band capable 			+ 1 pt.____

- □ Operator possesses a valid GMRS License and a Part 95 accepted portable capable of use on the 7 interstitial simplex frequencies common with FRS				+ 1 pt.____

- □ Part 95 Type accepted GMRS radio is also capable of repeater use		+ 1 pt.____

- Total Points Attained by RACES Member for this Vehicle (Pass 20)	____

- Vehicle is □ QUALIFIED □ NOT QUALIFIED   Evaluator Initial ________

- Radio Officer Review: □Approved □Appealed  □Denied Reason

- Radio Officer Signature:

- ______________________________ Call____________________